| Company / Organisation | Department: | Person performing Job: Service Engineer | Assessment by: D Tanner |
| --- | --- | --- | --- |
|  | Plant / Location: | Supervisor: Self Supervised | Reviewed By:J Box |
| Required and / or recommended Personal Protective Equipment (PPE): Ear protection, Safety Footwear, Safety Glasses, Overalls, Leather Palmed Gloves |  | Who may be harmed? Service Engineer | Approved by: J Box |

|  | Basic Task Step | Potential Hazard | Freq level | Prob level | Cons level | Risk FxPxC | Minimise Risk by (Control Measures) | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Using Analox Unit |  |  |  |  |  |  |  |  |
| Setup. |  |  |  |  |  |  |  |  |
| 1. | Place unit on suitable surface. | No significant hazard present. |  |  |  |  |  |  |
|  | Note: When testing compressor output, DO NOT place the monitor on the compressor, vibration from the compressor may cause the unit to fall. |  |  |  |  |  |  |  |
| 2. | Check sample hose for signs of damage. Replace hose if required. | No significant hazard present. |  |  |  |  |  |  |
| 3. | Connect sample hose to monitor, ensuring quick release coupling is fully engaged. | Finger pinch | 4 | 6 | 2 | 48 | Exercise care | Low |

|  | Basic Task Step | Potential Hazard | Freq level | Prob level | Cons level | Risk FxPxC | Minimise Risk by (Control Measures) | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. | Check that pressure source to be monitored is at between 5 and 345 barg. Connect sample hose to pressure source. If pressure source is compressor, start compressor. | Finger pinch | 4 | 6 | 2 | 48 | Exercise care | Low |
|  |  | Compressed air - Struck by projectiles, penetration wounds. | 4 | 6 | 4 | 96 | Ensure pressure source isolation valve is closed prior to connection. | Low |
| Purge Unit. |  |  |  |  |  |  |  |  |
| 1. | Switch unit on, and check display activates. | No significant hazard present. |  |  |  |  |  |  |
| 2. | Open pressure source isolation valve. | No significant hazard present. |  |  |  |  |  |  |
| 3. | Press the gas readings pushbutton. | No significant hazard present. |  |  |  |  |  |  |

|  | Basic Task Step | Potential Hazard | Freq level | Prob level | Cons level | Risk FxPxC | Minimise Risk by (Control Measures) | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Tests |  |  |  |  |  |  |  |  |
|  | Note: Ensure purging step has been carried out prior to purity testing. |  |  |  |  |  |  |  |
| 1. | Ensure gas readings are on display | No significant hazard present. |  |  |  |  |  |  |
| 2. | Read display and check the % Oxygen or Moisture ,co,co2 and voc, Dewpoint against standards. | No significant hazard present. |  |  |  |  |  |  |
| ## Oil Vapour Test |  |  |  |  |  |  |  |  |
|  | Note: Ensure purging step has been carried out prior to purity testing. Moisture dilutes the reagent system, and can affect the reading obtained. For this reason it is recommended to perform a moisture test prior to the Oil Test. |  |  |  |  |  |  |  |
| 1. | Take an oil detector tube, and break off both tips. Insert detector tube into the ‘Oil Test’ cell. | Sharp edges | 4 | 6 | 2 | 48 | Wear specified PPE Exercise care. | Low |
| 2. | Push the ‘Test’ pushbutton and run test for specified period. | No significant hazard present. |  |  |  |  |  |  |
|  | ## Note: Each time the ‘Test’ pushbutton is pressed the test will run for a period of 5 mins. Refer to manufacturers instruction sheet for required test period, and repeat ‘Test’ as necessary until total test time reached. |  |  |  |  |  |  |  |

|  | Basic Task Step | Potential Hazard | Freq level | Prob level | Cons level | Risk FxPxC | Minimise Risk by (Control Measures) | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Disconnection. |  |  |  |  |  |  |  |  |
| 1. | Close pressure source isolation valve. | No significant hazard present. |  |  |  |  |  |  |
| 2. | Switch Analox Unit on. | No significant hazard present. |  |  |  |  |  |  |
| 3. | Operate the purge valve to purge pressure from system. | No significant hazard present. |  |  |  |  |  |  |
| 4. | Wait until air stops ‘hissing’, then leave for a further minute. | No significant hazard present. |  |  |  |  |  |  |
| 5. | Disconnect hose from unit, and store in safe location. | No significant hazard present. |  |  |  |  |  |  |

Further Action Required.

| Step | Task | Hazard | Action Required |
| --- | --- | --- | --- |
|  |  |  | No further action required if existing risk controls used |

Document History

| Rev no: | Dated: | Reason for Update: | Updated by: |
| --- | --- | --- | --- |